CPE301 – SPRING 2019

# Design Assignment X

Student Name: Tian Chen

Student #: 2001086575

Student Email: chent5@unlv.nevada.edu

Primary Github address: https://github.com/Chentian12138/AAAABBBBB

Directory: DA5

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

We used NRF24L01 chip, atmega328p, and LM34.

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* main.c

*

* Created: 4/6/2019 9:49:11 PM

* Author : chent

*/

#ifndef F_CPU

#define F_CPU 16000000UL

#endif

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

#include <stdbool.h>

#include <stdio.h>

#include <string.h>

#ifndef BAUD

#define BAUD 9600

#endif

#include "inctest/STDIO_UART.h"

//Include nRF24L01+ library

#include "inctest/nrf24l01.h"

#include "inctest/nrf24l01-mnemonics.h"

#include "inctest/spi.h"

void read_adc(void);						//Read temp with the LM35 using ADC

void adc_init(void);						// Initialize ADC

void USART_init( unsigned int ubrr );		// Initialize USART communication

void USART_tx_string(char *data);			//sending string through usart

volatile unsigned int adc_temp;				// raw temp value from adc

volatile unsigned int tempF;				// convert to fahrenheit

volatile unsigned int tempC;				// convert to celsius

char outs[256];								// out for sending USART commands

volatile char received_data;				// String for receiving USART communication

//Used in IRQ ISR

volatile bool message_received = false;

volatile bool status = false;

int main(void) {

adc_init();								// Initialize Analog to Digital Converter on PC4

USART_init(BAUD_PRESCALLER);			// Initialize the USART

_delay_ms(500);							// Delay to sync all the commuication

nrf24_init();							// initial the nrf24

nrf24_start_listening();				// start listening from the nrf24

status = nrf24_send_message(tx_message);

if (status == true)

{

printf("Connection Established\n");

}

while(1)

{

PORTC^=(1<<5);						// port pc2 open

read_adc();							//read from adc

adc_temp = (adc_temp*500)/1023;		//calc the temp variable from the raw data

tempC = adc_temp;					// C value

tempF = (tempC*1.8)+32;				// F value

if(message_received)

{

// Message received, print it

message_received = false;		// Reset to false

printf("Received Temperature: %s\n" ,tempF, nrf24_read_message());

// Send message as response

_delay_ms(500);

// Perform ADC execution HERE and store in tx_message

status = nrf24_send_message(tx_message);

if(status == true)

{

printf("Temperature Transmitted\n");

}

}

_delay_ms(10000);

}

}

void adc_init(void)					//initialize ADC

{

ADMUX = (0<<REFS1)|				// Reference Selection Bits

(1<<REFS0)|						// AVcc - external cap at AREF

(0<<ADLAR)|						// ADC Left Adjust Result

(1<<MUX2)|						// Analog Channel Selection Bits

(0<<MUX1)|						// ADC2 (PC2 PIN25)

(0<<MUX0);						// 010

ADCSRA = (1<<ADEN)|				// ADC ENable

(0<<ADSC)|						// ADC Start Conversion

(0<<ADATE)|						// ADC Auto Trigger Enable

(0<<ADIF)|						// ADC Interrupt Flag

(0<<ADIE)|						// ADC Interrupt Enable

(1<<ADPS2)|						// ADC Prescaler Select Bits

(0<<ADPS1)|

(1<<ADPS0);

}

void read_adc(void) {

unsigned char i =4;

adc_temp = 0;						//initialize adc_temp variable

while (i--) {

ADCSRA |= (1<<ADSC);

while(ADCSRA & (1<<ADSC));

adc_temp+= ADC;

_delay_ms(50);

}

adc_temp = adc_temp / 4;			// Average a few samples

}

void USART_init( unsigned int ubrr ) {

UBRR0H = (unsigned char)(ubrr>>8);

UBRR0L = (unsigned char)ubrr;

UCSR0B |= (1 << TXEN0) | (1 << RXEN0)| ( 1 << RXCIE0); // Enable receiver, transmitter & RX interrupt

UCSR0C |= (1<<UCSZ01) | (1 << UCSZ00);

}

// Interrupt on IRQ pin

ISR(INT0_vect)

{

message_received = true;

}

void USART_tx_string( char *data ) {

while ((*data != '\0')) {

while (!(UCSR0A & (1 <<UDRE0)));

UDR0 = *data;

data++;

}

}

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- GITHUB LINK OF THIS DA

https://github.com/Chentian12138/AAAABBBBB

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT