CPE301 – SPRING 2019

# Design Assignment 6

Student Name: Tian Chen

Student #: 2001086575

Student Email: chent5@unlv.nevada.edu

Primary Github address: https://github.com/Chentian12138/AAAABBBBB

Directory:DA6

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega 328p and mpu 6050

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* DA 6.c

*

* Created: 5/5/2019 11:46:25 PM

* Author : chent

*/

#define F_CPU 16000000UL		// Board running at 16MHz

#include <avr/io.h>

#include <util/delay.h>

#include <inttypes.h>

#include <stdlib.h>

#include <stdio.h>

#include "MPU6050_res_define.h"		//

#include "I2C_Master_H_file.h"		//  Include necessary libraries

#include "USART_RS232_H_file.h"		//

double acc_X;	//

double acc_Y;	//

double acc_Z;	// Declare all variables for Accelerometer and Gyroscope

double gyro_X;		//

double gyro_Y;		//

double gyro_Z;		//

void MPU6050_Init()				// Gyro initialization function

{

_delay_ms(150);				// power up time

I2C_Start_Wait(0xD0);		// start with device write address

I2C_Write(SMPLRT_DIV);		// write to sample rate register

I2C_Write(0x07);			// 1KHz sample rate

I2C_Stop();

I2C_Start_Wait(0xD0);		// start with device write address

I2C_Write(PWR_MGMT_1);		// write to power management register

I2C_Write(0x01);			// Reference frequency of X axis gyroscope

I2C_Stop();

I2C_Start_Wait(0xD0);		// start with device write address

I2C_Write(CONFIG);			// write to configuration register

I2C_Write(0x00);			// Fs = 8KHz

I2C_Stop();

I2C_Start_Wait(0xD0);		// start with device write address

I2C_Write(GYRO_CONFIG);		// write to Gyro configuration register

I2C_Write(0x18);			// Celsius +/- 2000 degrees

I2C_Stop();

I2C_Start_Wait(0xD0);		// start with device write address

I2C_Write(INT_ENABLE);		// rite to interrupt enable register

I2C_Write(0x01);

I2C_Stop();

}

void MPU_Start_Loc()

{

I2C_Start_Wait(0xD0);			// I2C start with device write address

I2C_Write(ACCEL_XOUT_H);		// Write start location address from where to read

I2C_Repeated_Start(0xD1);		// I2C start with device read address

}

void Read_RawValue()

{

MPU_Start_Loc();														//

accelerometer_X = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());		//

accelerometer_Y = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());		//

accelerometer_Z = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());		//	read value of Gyroscope

gyroscope_X = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());			//

gyroscope_Y = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());			//

gyroscope_Z = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Nack());		//

I2C_Stop();

}

int main()

{

char buffer[20], float_[10];

float Xa,Ya,Za;

float Xg=0,Yg=0,Zg=0;

I2C_Init();						// initialize I2C

MPU6050_Init();					// initialize MPU6050

USART_Init(9600);				// initialize USART with 9600 baud rate

while(1)

{

Read_RawValue();

Xa = acc_X/16384.0;	// To obtain real values, divide raw value by scale factor

Ya = acc_Y/16384.0;

Za = acc_Z/16384.0;

Xg = gyro_X/16.4;

Yg = gyro_Y/16.4;

Zg = gyro_Z/16.4;

dtostrf( Xa, 3, 2, float_ );

sprintf(buffer," Ax = %s g\t",float_);

USART_SendString(buffer);

dtostrf( Ya, 3, 2, float_ );

sprintf(buffer," Ay = %s g\t",float_);

USART_SendString(buffer);

dtostrf( Za, 3, 2, float_ );

sprintf(buffer," Az = %s g\t",float_);

USART_SendString(buffer);

dtostrf( Xg, 3, 2, float_ );

sprintf(buffer," Gx = %s%c/s\t",float_,0xF8);

USART_SendString(buffer);

dtostrf( Yg, 3, 2, float_ );

sprintf(buffer," Gy = %s%c/s\t",float_,0xF8);

USART_SendString(buffer);

dtostrf( Zg, 3, 2, float_ );

sprintf(buffer," Gz = %s%c/s\r\n",float_,0xF8);

USART_SendString(buffer);

_delay_ms(1000);

}

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- GITHUB LINK OF THIS DA

https://github.com/Chentian12138/AAAABBBBB

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT